Version number: 1.1

Date: December 2016

Plextor Wi-Fi PTX Online Getting Started Guide

Font: Arial 18 pt

Produced by

Vision Australia Information and Library Service

Published by Vision Australia, 2016.

This work is copyright and copyright permission has been given to Vision Australia Information and Library Service to reproduce it for use by persons with a print disability.

Font: Arial 18 pt

Produced by

454 Glenferrie Rd Kooyong VIC 3144

Tel:	1800 005 965

Fax:	(03) 9864 9677

Email:	library@visionaustralia.org

Power Cord Maintenance Checklist

Important Information!

To ensure safe use of your player please carry out these simple checks on a regular basis.

- Turn off and unplug your player at the wall before checking that there are no twists or kinks in the power cord.

- Make sure the cord is never trapped in drawers, doors or underneath chair legs etc.

- Keep your power cords away from liquids and heat sources e.g. stove radiant heater etc.

- If you encounter any problems please call the Library immediately on 1300 654 656

Table of Contents

Topic 1: Introduction	4

Topic 2: Physical description of the Plextor Wi-Fi	6

Turning on the player	8

To hear Information about your books and player,	9

Key Describer mode	9

Topic 3: Playing a DAISY title	10

Rewinding and Fast forwarding	11

Heading Navigation feature	12

Returning a title from your bookshelf	12

To cancel a return	13

Topic 4: Using the Sleep timer feature	13

Topic 5: Troubleshooting	14

Contents of the box

- Plextor Wi-Fi Online DAISY Player.

- Power adapter with power cord.

- The Large print version of the Plextor Wi-Fi Player Getting Started guide.

- Laminated one page Quick Use guide including a diagram of the player on the reverse side.

- Internet cable (for back-up if required).

# Topic 1: Introduction

Welcome to the world of DAISY online! Thank you for registering to use our DAISY online players. We hope you will enjoy the over 30,000 DAISY audio books, 400 plus newspapers and magazines and selection of podcasts from Vision Australia radio.

The word DAISY is an acronym for Digital Accessible Information System. When an audio book is produced in DAISY format, the structure is similar to a printed book. This allows the reader more flexibility to navigate the audio book.

This Plextor Wi-Fi player has the ability to wirelessly connect via a modem to our online library and stream audio books, magazines and newspapers, without the need for the user to have access to a computer. The Wi-Fi player also provides functions that make it easier to navigate and use an audio book. For example, the built in function of remembering the last place read on a DAISY book. The Wi-Fi player can also play commercial audio books, music CDs and Mp3 files. You can also play content from a USB memory stick or an SD card.

You can have up to 5 books in your bookshelf at any time and borrow up to 5 new books per day. The books are selected automatically by the Library matching your reading preferences. When you have finished reading a book, simply return the book and a new book will be selected and added to your bookshelf the next day.

You can also choose your own books by logging in with your Library ID number and password to our Online Catalogue i-access Online:

www.i-accessonline.visionaustralia.org

Search for a book, subject or author. Choose a book by clicking on “Add to download queue.”

If your player is off, as soon as you turn it on, the books will appear.

If your player is on, you will need to turn your player off and on again OR hold down the 2 bookshelf arrow buttons together until it announces ‘bookshelf updated.’

Note: Books are on loan to you for 2 months.

The Wi-Fi PTX players are usually supplied pre-configured to your Wi-Fi network if you have provided the Library with your modem SSID and Password. If the player fails to connect to the library please phone the Library on: 1800 005 965 and ask for Jamie, who will help you configure the Wi-Fi settings over the phone.

Note: As an interim solution, you can use the supplied internet cable to connect the player directly to your modem.

For more information please contact the Library.

The following instructions are set out in a sequence of topics which will allow you to begin using the basic features of your Wi-Fi PTX player. Please take the time to follow these initial instructions to gain an understanding of the player’s functions.

If you would like information over the phone about using the Wi-Fi PTX player, please call the library.

# Topic 2: Physical description of the Plextor Wi-Fi

The Wi-Fi PTX player is sent with a mask attached. This mask can be removed if you want to use the player’s additional features. Please contact the Library before removing the mask and for further information.

The Wi-Fi PTX player is rectangular in shape. The player's top surface slopes down from the back to the front. The speaker grille is located on the top right hand corner of the player’s face.

The Power ON/OFF button is the round, depressed button, located below the speaker, on the right side of the player’s face. It is designed to be bump resistant.

Between the Power ON/OFF button and the speaker grille, is a small round button, this is the SLEEP TIMER. You can select from 15 minutes, 30 minutes, 45 minutes, 1 Hour, or OFF by pressing this button repeatedly. The player will then continue to play for that period of time before turning off.

Below the Power ON/OFF button is the RETURN button, this is a small square depressed button. Press this button to return a title from your bookshelf you have finished reading When you turn on the player the next day a new book will be available in the bookshelf.

Note: when playing a CD, press the Return button to eject the CD.

To the left of the Return button is the Title Key Left/Title Key Right toggle button. If you have multiple titles in your bookshelf you will use this button to move back and forward between titles.

Further to the left, at the bottom centre section of the player's face are a set of three buttons. From left to right, there is the REWIND button, the PLAY/STOP button and the FAST FORWARD button.

At the top of the player's face, there are two toggle buttons located side by side. From left to right, these are the TONE and VOLUME buttons. Pressing the top of each button will increase the tone or volume, and pressing the bottom of each button will decrease the tone or volume.

To change the volume quickly in either direction, press and hold the TONE and VOLUME buttons.

Press the up and down parts of the button together, to reset the volume and tone buttons to original settings.

On the left side of the player are 3 plugs or jacks; from front to back these are the SD card slot, the headphone jack and the USB socket. If you have DAISY content on a USB memory stick or SD card, you can play it on this player. For more information please contact the Library.

On the front edge of the player is the CD Insertion/Ejection slot. This CD loading mechanism differs from standard CD players, as there is no CD tray. You must have your player turned on first before you can actually insert or eject a CD.

## Turning on the player

The Wi-Fi PTX player needs a power outlet and will automatically shut down if the power cord is removed.

The power cord is in two parts which we have already connected together. To connect the player, plug the thin end of the cord into the small round jack in the centre at the back of the player. Plug the other end into a standard power point. Make sure you turn the power switch on at your wall socket.

To turn the player on, press and hold the Power ON/OFF button for 2 seconds until you hear a musical chime.

Note: When turning on the player for the first time, it can take up to 90 seconds for the player to connect to the Library and access your bookshelf. After a short pause the player will announce ‘Please wait’ and you will hear musical chimes until the player is ready to use. The player will then announce ‘No title from the bookshelf has been selected. Please press the title key to select titles from the bookshelf.’

Note: When you access your bookshelf for the first time, the only item in the bookshelf will be the Library’s Terms and Conditions. To loan books from the Library, you will need to read the Terms and Conditions. Then agree to them by pressing the Return button. The bookshelf will then update with a selection of books for you.

To turn the player off, press and hold the Power ON/OFF button for 2 seconds until the player announces ‘Power Off’. A Chime will sound as the power goes off.

Note: Do not use the Power ON/OFF button to temporarily pause the reading of a book. Use the rectangular PLAY/STOP button located at the centre bottom section of the player’s face. When you wish to continue reading the book, simply tap the PLAY/STOP button and the book will commence reading from exactly where you left off. If you want to turn the machine off, use the PLAY/STOP button to stop the book first, then use the Power ON/OFF button to turn the machine off.

## To hear Information about your books and player

Press and hold down the PLAY/STOP button for 2 seconds. Information such as the time elapsed, the time remaining, the total time of a book, heading information, power source, mode, firmware version and model specifications will be given.

## Key Describer mode

The Key Describer mode allows you to hear a description of a button’s function when pressed.  When you press the REWIND & FAST FORWARD buttons, Previous and Next title buttons and the Tone and Volume buttons, further help information is given.

The Key Describer mode is turned on by pressing and holding down the REWIND Button and FAST FORWARD button together for two seconds. To turn off Key Describer mode, again press and hold the REWIND Button and FAST FORWARD button together for two seconds.

When you have switched off Key Describer Mode, the player will announce ‘Key Describer off’ and a chime will be heard.

# Topic 3: Playing a DAISY title

Ensure the player is connected to a power point and switched on.

Select a title from the bookshelf by pressing the Title Key Left/Title Key Right button. The right key will select the next title and the left key will select the previous title. Title names are announced when selecting titles.

Note: Selecting a title after the player is first turned on will first connect to your Wi-Fi network and then announce ‘Bookshelf connecting to Vision Australia network updating the bookshelf.’

When you have begun to play a title, the player will automatically connect to the bookshelf next time it’s turned on, or to back to the CD if you were playing a CD.

Note: When you insert a CD, USB memory stick or SD card in the player, the items are added to your bookshelf.

To start playing the selected title, tap the PLAY/STOP key. The player will chime briefly before the title commences.

Note: When a title is played for the first time it will first announce ‘Beginning of title’ before the title commences.

If you wish to pause or stop reading, tap the PLAY/STOP button. A short chime will play. When you wish to continue reading the book, simply tap the PLAY/STOP button again and the book will recommence reading from exactly where you stopped it.

Note: From time to time the Library will send announcements to the player to inform you of updates to the service. Once you have heard the announcement, press the RETURN KEY and then the Play Key twice to delete the announcement.

## Rewinding and Fast forwarding

To REWIND and FAST FORWARD, use the Left and Right buttons either side of the PLAY/STOP button.

- Tap once to REWIND or FAST FORWARD to the previous or next heading. To REWIND or FAST FORWARD further than one heading, press and keep holding.  The REWIND or FAST FORWARD will skip 1 minute, 2 minutes, 3 minutes and so on in 1 minute increments. Playback will start when you release the button.

If the button is released before the player announces ‘2 minutes,’ playback will start at the place the button is released.

Note: The player will chatter for up to 10 seconds before the 1 minute increment is reached.

## Heading Navigation feature

The Heading Navigation feature is activated by pressing the FAST FORWARD or REWIND buttons through the main headings in a book or magazine.

Once the title of a book, newspaper or magazine has been announced, you can navigate through its headings using the FAST FORWARD and REWIND buttons.

Note: When reading, you can press the PLAY/STOP button to stop reading. You are then able to navigate through the book, newspaper or magazine’s headings by tapping the FAST FORWARD or REWIND buttons.

## Returning a title from your bookshelf

Press the return button, the button directly below the Power on/off button. You will hear the message ‘The title is reserved to be returned, press enter’ (which is the PLAY/STOP button) to confirm.’ When you press the PLAY/STOP button, you will hear ‘Return one title from the issue list, are you sure?’

Chimes play. Then press the PLAY/STOP button. You will hear ‘enter’ and then ‘One title has been returned from the issue list, updating the bookshelf. The bookshelf has been updated, select title.’

## To cancel a return

If you accidentally hit the RETURN button, press the RETURN button again and you will hear ‘Cancel the reservation of the return.’

Note: If the PLAY/STOP button has been pressed after the return button, pressing the REWIND button cancels the return.

# Topic 4: Using the Sleep timer feature

The Wi-Fi PTX player has a Sleep mode that allows you to set a delayed time period, after which the player will turn off automatically. The SLEEP button is the small button located under the speaker and above the Power ON/OFF button.

The Sleep mode has the following five options:

15 minutes

30 minutes

45 minutes

1 Hour

Off

Note: When the SLEEP button is pressed, the player will announce the set time delay. To turn the Sleep mode off, press the SLEEP button until your hear the ‘Off’ option announced.

To return to the position in a book before the sleep function was set, simply press and hold down the REWIND button until the appropriate time has elapsed. Press the PLAY/STOP button to commence reading the book.

# Topic 5: Troubleshooting

This topic outlines some of the problems you may encounter with your Wi-Fi PTX player and their possible solutions.

- If the player does not turn on, it may be because it is not connected to mains power. Re-insert the small round plug into the back of the player then connect the mains plug into an electrical outlet and turn it on.

- If the voice does not say anything, it may be because the volume is low. Increase the volume until you can hear the voice.

- If the PLAY/STOP button is pressed and no information is announced, the machine may not be set to SIMPLE mode. Please contact the Library so the player can be returned to the Library and reset to Simple mode.

- If the player does not move backward and forward between headings when the REWIND and FAST FORWARD buttons are pressed, the machine may not be set to SIMPLE mode. Please contact the Library so the player can be returned to the Library and reset to Simple mode.

- If the player announces ‘Failed to log on to the online service,’ press the Power ON/OFF button. When you turn the player back on, the problem will likely be resolved. If not, please contact the Library.

- If the player announces ‘No access points’ this means the player is not configured to your Wi-Fi network. Please contact the Library on: 1300 654 656 for help to configure the settings over the phone. Meanwhile you can still use the player online by using the supplied internet cable. Connect the cable to the internet socket on the back edge of the player to the left of the power socket and plug the other end of the cable in to an internet connection on your modem. Note: you can still play CDs on the player if the Wi-Fi settings are not configured.

- If a book won’t play and the player announces ‘Network error, connecting to Vision Australia’ the book maybe faulty. Please contact the Library and report the fault so the book can be fixed.

- If the player won’t shut down and keeps chiming, press the Power ON/OFF button and the Sleep button together and hold for 10 seconds then release the buttons. This will reset the player and shut down the player. Wait for 1 minute, then turn the player back on, the problem will likely be resolved. If not, please contact the Library.

Please review the instructions in this guide until you feel comfortable with using the common features of your player.

If you have any further questions about this information or using the Wi-Fi PTX player, please call the library on 1300 654 656

End of document