Version number: 1.0

Date: January 2009

VictorReader Stream Getting Started Guide

Font: Arial 18 pt

Produced by

Vision Australia Information and Library Services

Published by Vision Australia, 2009.

This work is copyright and copyright permission has been given to Vision Australia Information and Library Services to reproduce it for use by persons with a print disability.

Font: Arial 18 pt

454 Glenferrie Rd Kooyong VIC 3144

Tel:	1800 00 59 65

Fax:	(03) 9864 9677

Email:	i-access.online@visionaustralia.org

Table Of Contents

Contents Of The Box	2

Introduction	3

1. Turning The Player On	4

2. Changing Volume, Speed And Tone	4

3. Physical Description Of The Victorreader Stream	4

4. Key Describer Mode	5

5. Bookshelf Navigation	5

6. Navigation	5

7. Transferring Files Between Your Pc And The Stream	5

8. Using The Rewind And Fast Forward Functions	6

9. Go To Page	6

10. Bookmark	6

11. Info — Key 0	6

12. Using The Sleep-Timer Feature	6

13. Menu – Key 7	6

14. Where Am I? – Key 5	7

# Contents of the box

- VictorReader Stream.

- Power adapter with power cord.

- VictorReader Stream Quick Start guide audio CD (manufacturer).

- Large print version of the Victor Stream Getting Started guide (Vision Australia).

- Carrying pouch.

- Long USB cable to connect to the PC

- Short USB cable to connect to a USB memory stick.

- Earphones

- SD Memory Card

- Warranty Card

# Introduction

Welcome to the world of Daisy. Thank you for registering to use the VictorReader Stream, and we hope that you will enjoy the expanding collection of titles the library is offering in the Daisy format which has been adopted by blindness organizations world wide. The word Daisy is an acronym standing for Digital Accessible Information System.

When an audio book is produced in Daisy format, the structure can be similar to a printed book. This allows the reader more flexibility to navigate the audio book, for example, by page, chapter or heading.

The Victor Stream provides functions that make it easier to navigate an audio book. For example, the ability to set bookmarks and the built in function of remembering the last place read on a book.

The following instructions will allow you to begin using some of the most useful features of the Victor Stream. The manufacturer's User’s guide is also available.  You can access this by pressing and holding the 1 key on your Victor Stream.

# 1. Turning the player on

On the left side of the player, near the top corner, is the Power On Off/Toggle button. To turn the player on, press and hold the power button until you hear a long beep. To turn the player off, press the power button until you hear two short beeps.

# 2. Changing Volume, Speed and Tone

When the player is on, you press the Power button briefly to toggle the volume, speed and tone settings. Below the Power/Toggle button are two triangular buttons. These are the Up and Down buttons used to increase or decrease the selected setting.

(NOTE: In the Music folder instead of tone you will have treble and bass settings, and in the Text folder you have a pitch setting instead of tone.)

# 3. Physical description of the VictorReader Stream

On front there is a telephone style numeric key pad with a raised dot on the number 5 key. Above the 1 key is the Go To Page key. Above the 3 key is the Bookmark key.

There is a raised line below the numeric key pad and immediately below it, in the centre, is the Sleep-timer key. The bottom section contains the Play/Stop key, which is located between the Rewind and Fast Forward keys.

On the right side of the player, near the top corner, is 	the earphones jack which can also be used to connect an external speaker. Below it is the microphone jack. Halfway down the right side between two rough surfaces is the Record button which is red and has a raised dot in the middle.

Located on the top end of the player, on the left side is a USB port. In the middle, is the SD memory card slot. On the right is the power input jack.

# 4. Key Describer mode

When no SD card or USB source is present, the Key Describer mode will be activated automatically.

# 5. Bookshelf navigation

The Stream recognizes various book types which are saved in separate folder structures called Bookshelves (for example: Daisy Talking Books, Text Files, Music). You select the Bookshelf you want with multiple presses of the Bookshelf ( number 1) key. You move backward and forward along each Bookshelf using the number 4 and 6 keys.

# 6. Navigation

The Stream allows you to navigate by chapter, section, page, time jump, paragraph, or any other existing indexed element defined by the producer of the book. Use the number 2 and 8 keys to choose navigational level. Then use the number 4 and 6 keys to move backward and forward between elements at the selected level.

# 7. Transferring Files Between Your PC and the Stream

To connect the Victor Stream to your PC, ensure the power is turned off on the Stream. Connect one end of the long USB cable to the USB port on the top left edge of the Stream and the other end to your PC's USB port. Then turn the Victor Stream on. Windows will recognize the Victor Stream SD card as a USB drive and you can then use Windows Explorer to transfer files back and forth. To disconnect the Stream, use the “remove hardware safely option and then turn the Stream off before restarting it to listen to your books.

# 8. Using the Rewind and Fast forward functions

These keys allow you to go backward or forward quickly.  The Rewind or Fast Forward keys will accelerate the longer you hold the key down. For each three seconds you hold the keys down you will move forward or backward 1 minute, 2 minutes, 5 minutes, 10 minutes, 15 minutes and so on.

# 9. Go to Page

Press the Go to Page key, above the number 1 key, then enter the desired page number followed by the Pound key.

# 10. Bookmark

Bookmarks allow you to return to a place quickly and easily. You can place a virtually unlimited number of bookmarks in the same book. The Bookmark key lets you insert and return to bookmarks, hear them listed or remove them.

# 11. Info — Key 0

The Info (number 0) key announces book, player, memory card and battery information.

# 12. Using the Sleep-timer feature

The Stream has a Sleep-timer mode that allows you to set a delay time period of 15, 30, 45 or 60 minutes, after which the player will turn off automatically.

# 13. Menu – Key 7

Press the Menu key (number 7) to browse menu items related to the player set up. Use the Up/Down (number 2, 8) keys to move through the list and use the Left/Right (number 4, 6) keys to select an Item’s value and press Pound or Hash key to confirm. Press Cancel (star key) or any other key to exit the Menu.

# 14. Where Am I? – Key 5

The Where am I (number 5) key will announce your reading position without interrupting the reading process. Depending on the book type, the Stream may announce; the page and heading numbers, section title or file name. For Other books, music and podcasts, Where am I will announce the individual file’s total time, elapsed time and time remaining. Reading will resume from the previous point once this information is given.

15. For more detailed information listen to the Manual on the Victor Stream by pressing the number 1 Key for 2 seconds. To exit the manual, press and hold the number 1 key again for 2 Seconds.

You can also visit Humanware’s website for the VictorReader Stream documentation.

http://www.humanware.com/en-canada/support/vr_stream_documentation

End of document.